### Troubleshooting Steps – How to find the problem.

- On R1 CLI, run:

show ip interface brief

You will see Serial0/0/1 is administratively down.

- Check the routing table:

show ip route

You will notice the EIGRP/OSPF redistributed routes are missing.

- Run extended ping to diagnose path:

ping 30.30.30.1 source 10.10.10.1

Why Failure Occurred?

In the lab, the fault was introduced by running the shutdown command on the serial interfaces — specifically Serial0/0/1 on R1 and Serial0/0/1 on R2.

The physical links went down. When you shut down those interfaces, R1 lost its direct serial link to R3, and R2 lost its link to R3. Both paths into the R3 LAN were severed.

Routing protocols lost their neighbours. OSPF (between R1 and R2) and EIGRP (between R1/R2 and R3) both rely on those physical links to exchange routing updates. Once the interfaces were administratively down, the OSPF and EIGRP neighbour relationships dropped — meaning the routers stopped sharing route information entirely.

Redistributed routes disappeared. R1 was responsible for redistributing EIGRP routes into OSPF and vice versa. With no EIGRP neighbour reachable, there were no EIGRP routes to redistribute. So R1's routing table lost the entry for 30.30.30.0/24 completely.

The result: PC1 sent a packet to 30.30.30.30, R1 looked in its routing table, found no matching route, and dropped the packet — causing the "Request timed out" ping failure.

In short: no interface = no neighbour = no route = no connectivity. This is also why show ip interface brief and show ip route are the two most important troubleshooting commands — the first shows you the physical problem, and the second shows you the routing consequence.